HDNet Movies and AXS TV File Delivery Information

Effective June 1, 2022

File Specifications

Format Name: MPEG HD422 (XDCAM HD)

Container: MXF

Operational Pattern: OP-1A

Video Coding: MPEG-2 422P@HL

Bit Depth: 8

Color Sampling: 4;2:2

Frame Size: 1920 x 1080

Frame Rate: 29.97

Scanning Type: Interlace

Video Bit Rate: 50 Mbps (CBR)

Insert after Frame Rate

Field Order: Upper First

Audio: Channel 1 and channel 2 – Stereo or Mono (native to movie); 5.1 Surround Sound Audio Mapping:

Ch. 1 – Front Left

Ch.2 – Front Right

Ch.3 – Centre

Ch.4 – LFE (ie. bass)

Ch. 5 – Rear Left

Ch. 6 – Rear Right)

Captions:  embedded on 608/708 (please send separate .scc file)

Tech Contact: ittechs@axs.tv

Administrative Contact:  Diana Ritter, dritter@hdnetmovies.com